Name_____________________________________________________

Activity 4.3.3 Air System

Purpose

Some agricultural machines have fluid power systems that an operator needs to control from one location. The design of the machine may not allow an operator to manually open and close valves controlling fluid power. How else can an operator control a fluid power system?

Solenoids are electronic devices using electromagnetic current to operate a switch controlling valves in a fluid power system. Operators use electronic controls to send electricity to the solenoid. The electrical current causes the solenoid to become magnetic. The magnetic properties of the solenoid then move the valve to a different position diverting pressurized fluid to a cylinder actuator.

Throughout Lesson 4.2 Control Systems, you programmed a PLC to control motors and interpret data from sensors. The VEX cortex can be programmed for a touch sensor to control solenoids activating pneumatic valves. When a solenoid has a value of “1” in a ROBOTC program, it is activated. A solenoid with a value of “0” is off. Programmers use if-else and while-loop statements to program a touch sensor to activate solenoid valves in a pneumatic system.

What is the process for programming and operating a solenoid in a fluid power system?

Materials

| Per team of three students: Computer with ROBOTC software VEX Cortex Touch sensor USB cable 3-wire adapter | Per student: Agriscience Notebook Engineering Notebook Pen PPE |
| --- | --- |
| 7.2V battery Accumulator Double acting cylinder Two-position, solenoid-operated, spring-return, thee-way valve Two-position, manually operated, two-way valve Air pump Pressure regulator Five 6″ pieces of 4mm tubing |  |

Procedure

Observe your teacher’s demonstration on how to attach and detach hoses from system components. Then construct a pneumatic system controlled by a solenoid. Select a teammate in your group to complete Part One. The other two will complete Part Two. Work as a team to complete Part Three.

Part One – Program

| Open ROBOTC. Rename dgtl7 as Solenoid and change Sensor Type to Digital Out. Rename dgtl8 as touchSensor and change Sensor Type to Touch. Enter the program shown in Figure 1 into ROBOTC. Add comments to the program explaining behaviors. Save the program as 4_3_3PneuYourInitials. Connect the computer to the cortex using a USB cable. Select Robot and change the VEX Cortex Communication Mode to USB Only. Download the program to the cortex. |  |
| --- | --- |
|  | Figure 1. Solenoid Program |

Part Two – Assembly

| - Put on PPE and tie back hair. - List all components shown in Figure 2 in your Engineering Notebook. - Use the schematic in Figure 2 to assemble the pneumatic system. Note the direction of air flow for the regulator, two-way valve, and four-way valve shown in Figure 3. - Use a 3-wire adapter to connect the solenoid to dgtl7 of the cortex. - Attach the touch sensor to digtl8 of the cortex. - Attach the 7.2V battery to the cortex. Part Three – Operation - Close the two-way valve. - Turn the screw on the air regulator counter-clockwise to close the regulator and not allow air into the system. - Connect pump to the accumulator. |  |  |  |
| --- | --- | --- | --- |
|  | Figure 2. Schematic |  |  |
|  | Regulator | Two-Way Valve | Four-Way Valve |
|  | Figure 3. Valve Air Flow |  |  |

- Use the pump to compress air into the accumulator until the pressure is 55psi. It will take approximately 5 – 8 pumps.

Disconnect pump from the accumulator.

Listen for air leaks between the accumulator and regulator connections. Tighten fittings if air is leaking.

- Turn the air regulator screw clockwise to open the regulator and allow air into the system.

Listen for air leaks between the regulator and two-way valve connections. Tighten fittings if air is leaking.

Open the two-way valve.

Listen for air leaks between the two-way valve and solenoid valve connections. Tighten fittings if air is leaking.

Turn on the cortex and wait for the green lights to blink indicating the cortex is ready.

Press and depress the touch sensor button and observe the cylinder.

Troubleshoot the following in your system if the cylinder actuator does not extend and contract.

Pressurized air in system

Air leaks

Electrical connections

Battery power

Programming

Have your teacher check the system for proper operation.

Store your system as instructed by your teacher. You will use it during the next activity.

Conclusion

- What are the advantages of a solenoid operated valve?

- Where would a manually operated valve be on agricultural equipment?

- Where would a solenoid valve be used in the agricultural industry?