Name_____________________________________________________

Activity 4.3.4 Fluid Flow

Purpose

When watching an assembly line in action, you will see cylinder actuators doing tasks that require a large amount of force and other tasks requiring slow, precise movements. How do technicians control force and speed of cylinder actuators in a fluid power system?

You have used valves opening and closing fluid passageways. Valves can also control the pressure and flow of fluid moving through a system. Pressure regulators control the air pressure, measured in pounds per square inch (psi), in a fluid power system. Technicians use regulators to maintain a constant pressure throughout a system. The pressure in the system may be lower than air pressure provided from the accumulator.

Flow control valves control the rate of fluid flow in a system. The flow rate affects the speed of actuators. Slowing down the speed of an actuator allows an operator to have precise control. Flow control valves may have check valves allowing fluid to flow in only one direction. Flow controls prevent fluid from flowing backwards into a system and maintain pressure in the system after fluid flow stops.

Materials

| Per team of three students: Assembly from Activity 4.3.3 Air System Plastic T fitting Three 3″ 4mm pieces of tubing Two flow control valves Flow control tool LabQuest2 Pressure sensor Force sensor | Per student: Agriscience Notebook Engineering Notebook Pen PPE |
| --- | --- |

Procedure

Work with your teammates to assemble fluid systems with a regulator and flow control valve. Then measure the force of the cylinder’s actuator at various pressures and flow settings.

Part One – Pressure Sensor

| Put on PPE and tie back hair. Place one 3″ piece of tubing into each end of the T fitting as seen in Figure 1. Replace the tubing between the regulator and two-way directional control valve with the T fitting and tubing. Use the schematic in Figure 2 as your guide. Connect remaining open end of the T into the pressure sensor. |  |
| --- | --- |
|  | Figure 1. T-fitting Attachment |

| Turn on the LabQuest2. Attach the pressure sensor to Channel 1. Change the units to psi. Zero out the reading. Attach the force sensor to Channel 2. Place the rubber stopper on the end of the force sensor. Change the units to pounds. Zero out the reading while holding sensor horizontally Change the reading to reverse. |  |
| --- | --- |
|  | Figure 2. Schematic with Gauge |

Make a table in your Engineering Notebook with three columns titled system pressure, actuator force, and actuator speed. Add three rows under each titled column.

Close the two-way valve.

Loosen the regulator so no air flows through the pressure gauge.

Use the air pump to pressurize the accumulator to 85psi. It will take approximately 5 – 8 pumps.

Open the two-way directional valve.

Tighten the regulator to allow air into the system until the pressure sensor reads 15psi.

Turn on the cortex.

Operate the cylinder using the touch sensor and observe the cylinders behavior.

Record the system pressure in the first row.

Describe the speed of the actuator in your Engineering Notebook.

While holding the touch sensor with the actuator extended, use the force sensor to push the actuator half-way in.

Hold sensor and cylinder horizontally while testing.

In the table in your Engineering Notebook, record the force needed to move the actuator

Tighten the regulator to allow air into the system until the pressure sensor reads 30psi.

Repeat Steps 14 – 15.

Tighten the regulator to allow air into the system until the pressure sensor reads 45psi.

Repeat Steps 14 – 15.

Close the two-way directional valve and operate the cylinder to release the pressure from the system.

Rank the speed of the actuator under different pressures from fastest (1) to slowest (3) in the table.

Part Two – Flow Control

| - Disconnect the tubing from the ports on the double acting cylinder. - Replace the ports on the double acting cylinder with flow control valves as shown in Figure 3. - Connect the tubing to the flow control valves on the double acting cylinder. |  |
| --- | --- |
|  | Figure 3. Flow Control Valves |

Make a table in your Engineering Notebook with four columns titled flow control setting, system pressure, actuator force, and actuator speed. Add three rows below each column title.

Tighten the regulator to allow air into the system until pressurized at 45psi.

Operate the cylinder using the touch sensor and observe the cylinder’s behavior with fluid flow at the standard setting.

Use the force sensor to push the actuator in while extended.

Record the flow control setting, system pressure, and force needed to move the actuator in your Engineering Notebook.

Describe the speed of the actuator in your Engineering Notebook.

Use the valve tool to loosen the flow control valves two revolutions to the left to decrease fluid flow.

Repeat Step 6.

Tighten the screw on the flow control valves four revolutions to the right to increase fluid flow.

Repeat Step 6.

Return the flow control valves to the original positions by loosening each two turns to the left.

Rank the speed of the actuator under different flow control settings from fastest (1) to slowest (3) in the table.

Disassemble your systems and store away materials as instructed by your teacher.

Conclusion

- How is fluid pressure and flow used to control cylinder actuators?

How are fluid pressure and flow different?

When would an operator want to increase pressure in an agricultural system?

- When would an operator want to decrease flow in an agricultural system?