Name_____________________________________________________

Project 4.3.1 Threshing Machine

Purpose

All food products you consume have been processed into a form you eat. The process for producing an edible food product may be as simple as harvesting the food and washing it before consumption. Other processes are more complex requiring mechanical cutting and grinding to convert an agricultural crop into a food product. How are machines designed to process food?

Food processing systems consist of the same basic components you worked with in previous lessons. A mechanical processing system will have an energy source, drivetrain, manipulators, and control system. The drivetrain has components transferring power from one location to another. Synchronous drivetrains can contain gears, sprockets, and chains transferring power with no slippage between the parts. Gears interlock together while a roller chain connects sprockets. In a synchronous drive system, a drive gear or sprocket attached to the drive shaft of a motor turns the driven gears or sprockets which do work.

A combine harvester is a complex machine with shafts facing multiple directions moving at various speeds. The driven shafts operate a threshing system separating the grain from a plant. Power from a single drive shaft from the engine is transferred to systems of different gears and drive components to thresh the grain. How is the threshing system in a combine designed to process and harvest grain?

Materials

| Per team of students: VEX robotics gears and parts Computer with ROBOTC VEX Cortex Touch sensor 7.2V battery USB cable Fabrication tools Measurement tools | Per student: Computer with internet access PPE Agriscience Notebook Engineering Notebook Project 4.3.1 Evaluation Rubric Employability Evaluation Rubric Pen |
| --- | --- |

Procedure

Work individually to research the mechanical threshing system and components in a combine. Explain the purpose of each component. Then identify the driven rotary and linear motions driving those components. Record the direction and speed of each motion. Listed below are example components working in a combine harvester to help start your research. Refer to the diagram on page 583 of Foundations of Engineering and Technology textbook. Record all research and sketches in your Engineering Notebook.

Reel

Cutter bar

Conveyor

Rotor drum

Grain auger

Straw walker

When instructed by your teacher, meet with your assigned teammates to design and construct a threshing system powered by a single motor in a drive system. The system should include a working model along with speed calculations and detailed sketch recorded in your Engineering Notebook. You will have access to all drive system components discussed during the Drive Systems presentation. The model should meet the following criteria.

Use one motor to supply the power

A minimum of four motion variations simulated

Three rotary motions

One linear motion

Calculated gear ratios, gear reductions, and speeds

System programmed to turn on and off with a touch sensor

Your teacher will inspect your assembly to ensure you can operate it safely meeting the following criteria.

Turned on and off with a touch sensor

All components securely fastened

Has no interfering parts

The model, recorded research, sketches, and calculations will be assessed using Project 4.3.1 Evaluation Rubric. Your teacher will use the Employability Evaluation Rubric to assess your work behavior during the project.

Conclusion

What is the purpose of using different types of gears and sprockets in a mechanical system?

What challenges do engineers face when designing agricultural systems?

What factors affect the types of components used in a drivetrain?