Name_____________________________________________________

Project 4.3.5 Assembly Line

Purpose

You may have put together a model or Lego® set by hand. Imagine the planning it would take to design a machine to assemble the model. What type of planning and work would it take to construct a machine to put a Lego® set together? A mechatronics system is an automated machine programmed to complete manufacturing processes similar to assembling a Lego® set. Mechatronic systems manufacture or process most agricultural products we use today. Mechatronics combines the mechanics of a machine with the programming of a computer. What are some components of a mechatronics assembly line?

Fluid and electricity power the machines found in a mechatronic system. Electrical motors power conveyors and rotary machines to transport, cut, and process a product. Pneumatic cylinders control robotic arms, clamps, and tools used to complete each step of the assembly process.

An assembly line may need to start and stop many times during the manufacturing process, so workers and machines can complete specific steps. Sensors on the assembly line detect the environmental surroundings causing the line to stop and machines to complete the work. Multiple sensors controlling movements need to be programmed so all moving parts work in harmony with each other and do not harm any workers on the assembly floor. When more than one sensor is controlling multiple movements, a more complex program must be written. Figure 1 is an example flow chart and pseudocode showing how If/if-else statements instruct a robotic tractor to stop if it is dark or within five feet of an object.

|  | if (light outside) |
| --- | --- |
|  | { |
|  | tractor moves forward at half speed |
|  | if (tractor has no object within 5 feet) |
|  | { |
|  | tractor moves forward |
|  | } |
|  | else |
|  | { |
|  | tractor stops |
|  | } |
|  | } |
| Figure 1. Example Flow Chart and Pseudocode |  |

What steps must an engineering team take to have a computer, mechanical, and pneumatic system work together on an assembly line?

Materials

| Per team of three students: VEX mechatronics materials VEX sensors VEX pneumatic materials Flow control tool VEX Cortex Computer with ROBOTC USB cable 1.5″ wood block Fabrication tools Measurement tools | Per student: Agriscience Notebook Engineering Notebook Engineering Report Template Engineering Report Evaluation Rubric MSA Notebook Evaluation Rubric Pen PPE |
| --- | --- |

Procedure

Work in a team of three to design, program, and construct a model of an assembly line for manufacturing a product.

Part One – Individual Brainstorm

Design an assembly line consisting of a conveyor which starts and stops two times. The first stop is to inspect the product. Inspection takes five seconds. The second stop is to drill a hole in the top of the product. After the product is inspected and drilled, it is ejected off the assembly line into a bin. The bin is on the side of the conveyor due to limited space. In summary, the model assembly line will perform the following tasks.

Transport a 1.5 inch woodblock a minimum of 8 inches

Stop the transport for five seconds for inspection

Stop the transport for drilling and ejection

Simulate the downward motion of a drill

Eject the woodblock off the conveyor

Work individually to sketch your assembly line ideas in your Engineering Notebook. Include a list of sensors, motors, and pneumatic actuators you would use in your system, along with a flow chart explaining the process.

Part Two – Share and Select Idea

Meet in your assigned team and share your ideas. Develop a consensus among your team on which plan or combination of plans you will use to design and construct the assembly line.

Part Three – Assign Tasks

Assign a tradesperson, technician, and engineer for your team. Review the assigned tasks in Table 1 for each teammate. Record all notes, procedures, and sketches related to your assigned task in your Engineering Notebook.

| Table 1. Engineering Roles |  |  |
| --- | --- | --- |
| Tradesperson | Technician | Engineer |
| Construct assembly line Work with technician to determine layout and schematic Attach assembly line sensors, motors, and solenoid valves to the cortex Record assembly problems | Record materials and construction procedures Sketch schematic and label major components Assist engineer with flow chart development and pseudocode | Develop flow chart explaining the assembly process Convert flow chart to pseudocode Enter programming code into ROBOTC Download code to cortex |

Part Four – Operate and Troubleshoot

After constructing and programming the assembly line, work in your team to complete the following tasks. Record all procedures and information pertinent to those tasks in your Engineering Notebook.

Operate, test, and troubleshoot the assembly line for proper function.

Make changes to construction or program to improve operation.

Part Five – Engineering Report

Work in your team using the Engineering Report Template to complete an engineering report explaining your design. Turn in your Engineering Notebook and the team’s engineering report to your teacher. Your Engineering Notebook will be assessed using the MSA Notebook Evaluation Rubric and your team’s report will be assessed using the Engineering Report Rubric.

Conclusion

How is mechatronics used for assembling machines and products?

What types of skills do workers need in the mechatronics field?

Why are assembly lines used in agricultural manufacturing and processing?