	[image: image3.jpg]F

Re

I
g

ELANDS

ional Medical Center




	Title:  Insulin Pen Devices for Inpatients  
	ID #: 

Effective: 1/08

Page: Page 1 of 3

	
	STANDARD POLICY AND PROCEDURE FORM
	

	Written By:  Clinical Pharmacy Manager
	Distribution:  Nursing Policy & Procedure 

	
	Manual, Pharmacy Manual

	Approved By (signature & title): 
	Revised:  3/09, 10/11

	Senior VP & CNO
	

	
	Reviewed:  4/10

	
	


PURPOSE:   To effectively establish a method for the secure storage and safe and 
   efficacious use and education of premixed insulin pen devices on the inpatient units in 

   accordance with best practice guidelines.
PROCEDURE:   The pharmacy department will dispense the following patient specific insulin pens to patients on the inpatient units (4C, 4P, 4N, 3T, 3N, 1S, PEDS, REHAB)*:

-Humalog KwikPen (28 day expiration at room temperature)
-Humulin N Pen (14 day expiration at room temperature)
-Humalog Mix 75/25 KwikPen (10 day expiration at room temperature)

-Lantus (glargine) SoloSTAR Pen (28 day expiration at room temperature)

* Nursing should pay special attention to the labeled expiration date of the insulin pen supplied and contact the pharmacy department for a refill when necessary)

In addition to the above patient-specific insulins, the pharmacy department will supply (and charge to floorstock) all outpatient and procedural areas unit with an emergency supply (1 vial of each) of the following insulin vials:

Humalog, Humulin R, Humulin N, Humalog 75/25, and Lantus.  All insulins drawn up from multiple-dose vials are considered high-risk medications and must be double checked.  Double checks are required prior to administration of insulin pens.  Both nurses will sign the MAR to verify double check procedure. 

These vials will be properly dated when opened in accordance with nursing policy (See Multi Dose Vial Policy) and stored in the refrigerator on the nursing unit in separately labeled high risk medication bins to prevent medication errors.  Examples of when these floor-stock insulins may be used include, but are not limited to:


-Use of short-term insulin or single doses in the outpatient care areas


-Patient transfer from main campus and due for insulin on arrival to rehab unit


-Any other emergent situation  

If at any time the patient or unit requires a refill of insulin, please notify the pharmacy using the yellow medication communication form.
*All patients on an insulin pen and all patients new to insulin should be properly educated (referrals to diabetes education are strongly encouraged whenever feasible in these circumstances).

	[image: image1.jpg]=l FIRELANDS
.' REGIONAL MEDICAL CENTER




	Title:  Insulin Pen Devices for Inpatients
	ID #: 

Effective: 1/08

Page: Page 2 of 3

	
	STANDARD POLICY AND PROCEDURE FORM
	


ADMINISTERING INSULIN VIA A PEN FILLED DEVICE:

Preparing the Insulin Pen

1.) Remove the pen from the patient’s medication drawer.  For each injection, wash your hands, wear protective gloves and practice sterile technique.  Remove the pen cap and be sure to check pen labeling for type of insulin, expiration date and appearance.  Use an alcohol swab to wipe the rubber seal on the end of the pen. 
2.) 
HumaLog Mix 75/25 and Humulin N are suspensions and should be visually inspected 
     and resuspended immediately before use. The re-suspended solution must appear 
uniformly white and cloudy. Before initial use, (DO NOT SHAKE) roll the disposable 
prefilled syringe pen between your palms 10 times. Thereafter, turn the disposable 
syringe upside down so that the glass ball or bubble moves from one end of the reservoir 
to the other. Do this at least 10 times. The rolling and turning procedure must be 
repeated until the suspension appears uniformly white and cloudy. Inject immediately. 


3.) Remove the protective paper tab and twist a BD Ultra-Fine III needle onto the insulin dosing pen until tight.  Always place a new needle on the insulin pen for each injection.  
Priming the Insulin Pen

4.) Remove the outer needle shield (do not throw away).  Remove the inner needle shield and throw away.  

5.) Turn the dose dial to 2 units and hold the pen so that the needle faces up in the air.  For Humulin N pen only: make sure arrow is in center of window and pull dose knob out.  Then turn the dial to 2 units.  

6.) Gently tap the top of the syringe to move any air bubbles to the top of the pen.  
7.) Push down on the dosing button and see if a drop of insulin appears.  If not, repeat this procedure (called an airshot) until a drop appears.  If you do not see insulin expelled at the needle tip and the dose knob becomes hard to push, change the needle and prime the pen once again.  If you ever need to make more than 6 airshots, do not use the syringe and contact the pharmacy for replacement.  Pharmacy shall contact the manufacturers for any reports of syringe failure.  
Injecting the Insulin Dose
8.) Before setting the dosage, make sure that there is enough insulin remaining in the reservoir to administer the correct dosage (you cannot set a dose greater than the number of units remaining in the reservoir).  
9.) Turn the dosage dial to set the dose selector to the desired dosage (in units).  If you turn the dial further than the dosage desired, simply turn the dial back to the correct dosage.  For Novolin N pens, make sure arrow is showing in window and pull dose knob out before dialing the dosage.

	[image: image2.jpg]=l FIRELANDS
.' REGIONAL MEDICAL CENTER




	Title:  Insulin Pen Devices for Inpatients
	ID #: 

Effective: 1/08

Page: Page 3 of 3

	
	STANDARD POLICY AND PROCEDURE FORM
	


10.) Insert needle into skin using standard nursing procedure for insulin administration

        (Refer to 2008 Clinical Nursing Skills Book, pg. 610 – 613).  
11.) Once the needle is inserted into the desired subcutaneous site, press the push button  

        all the way in.  The dosage scale will then reset to zero (0) or a diamond will show in the 
window for the Novolin N pen.  With the push button fully depressed, keep the needle in 
the skin for at least 5 seconds to ensure the full dose has been delivered.  Keep the 
push button fully depressed until the needle is withdrawn from the skin.  
12.) After injection, remove the needle from the skin, carefully replace the outer needle 

        shield and unscrew and discard the needle in the nearest sharps container.  Return to
        the patient’s medication drawer and chart administration.  Do not store the pen with
        the needle attached. 
