Four Moments In Time – Daily Audit

There are four moments in time where the physical inventory record must be equal to the computer record.  If these four opportunities are audited daily and reconciled, most variance will be eliminated and routine bin checks and physical inventories will only be necessary to detect fraud. The four opportunities are:

- When the computer record quantity is reduced to zero, the physical condition should also be checked to make sure it is also zero.

This condition is reported in the Out of Stock Report.  It may be named differently on different dealer computer systems, but the underlying tenants of the reports are the same.  The report displays all part numbers that had the quantity on hand reduced to zero in the previous days business.  A sample report is shown below.

Four Moments In Time – Daily Audit (continued)

- When the physical quantity on hand is reduced to zero, the computer quantity on hand should be checked to see if it is also zero.

This condition is reported on the Zero Stock Board.  The Zero Stock Board is a manual record that is kept at the parts counter and maintained by the people at the counter.  The counterperson records the part number on the zero stock board when the last part in the bin is handed out and the physical quantity remaining is zero.  A sample of a Zero Stock Board is shown below.

Four Moments In Time – Daily Audit (continued)

- When the computer record is reduced to less than zero, the physical transaction should be checked to determine if the negative is “false” or if the transaction was posted to the wrong part.

This condition is reported on the Negative On Hand Report from the dealer computer system.  The report needed here is the report that displays all parts that were reduced to a negative quantity as a result of yesterday’s business.  Many of the Negative On Hand reports in different dealer computer systems show the cumulative result of all parts reduced to a negative quantity for the entire month.  This information is too hard to track, so we must have a report that shows the results from yesterday’s business.  This makes it easier to reconcile with the other opportunities on a daily basis.  A sample report is shown below.

Four Moments In Time – Daily Audit (continued)

- When the computer suggests an order for a part, it should be determined that the computer record upon which the order is calculated is the same as the physical condition.

This condition is detailed on the suggested stock order from the dealer computer system.  The actual stock order report is formatted differently on the different dealer computer systems, but the basics of the report are similar across all lines.  The previous reports will determine variance in all parts that were reduced to zero or negative.  There are other parts that have not been reduced to zero but the computer recommends an order for the part based on our depth criteria.  For instance, you computer best stocking level for a filter is ten filters.  You have sold down to six in inventory since your last stock order and the computer recommends that you purchase four more.  The computer recommends purchasing four because it “thinks” you have six in inventory.  If the actual count is different than six on hand, the computer will order incorrectly.  All stock order purchase recommendations for parts that have a current quantity on hand greater than zero need to be checked for accuracy to insure proper ordering.

The Daily Database Audit Procedure

When you are conducting the Database Audit every day, you are combining the Zero Stock Boards, the Out of Stock Report, and the Daily Negative On Hand Report into one Management Process.  Once you have reconciled the Zero Stock Boards with the Out of Stock Report, and crossed off any parts that appear on both reports, the parts that remain on the three reports fall into one of three scenarios.  Use the guides on this page to assist you in determining why a part appears here, and what may have caused a variance.  After researching the cause of the variance, make an effort to correct the cause of the variance.

| Condition One: Part appears on ZSB, but not on ROS. - Part is NS status. - Most recent days sale not posted because part is set aside for customer, or on special order shelf. - Transaction was posted to the wrong part. - Sale was posted to a superseded part number. - Wrong quantity receipt posted. - Floor denials, backorders, shortages not posted to inventory. - Requests for credit or factory return not posted to inventory record. No Resolution? Make the appropriate Minus Adjustment | Condition Two: Part appears on ROS, but not on ZSB - Counterman forgot to write part number down. - Transaction posted to wrong part number. - Credit memo posted as sale. - Part put in the bin, but not receipted yet. - Remanufactured part sold under original part number. - Sale posted to superseded part number. - Package quantities posted incorrectly. - Unsold body shop parts sold second time. No Resolution? Make the appropriate Plus Adjustment . | Condition Three: Part listed on Negative on Hand Report - Sale posted prior to receipt posting. - Transaction posted to wrong part number. - Package quantity posted incorrectly. - Long Number part sold under Short Number. - Credit memo posted as a sale. - Parts received in kit but sold as pieces. - Unsold body shop parts sold a second time. - Sale posted under superseded part number. No Resolution? Do nothing until the part has been negative one full week, and then make the appropriate Positive Adjustment, if still necessary |
| --- | --- | --- |

Notes

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________